LINEA Thomson-PlasmonX Preliminary layout for total cost estimation

Thomson-PlasmonX common beamline

| Layout Thomson (E= 28 ÷ 150 MeV) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Element | Type | Length (mm) | Field gradient (T/m) |  | Theta (º) | B (T) | Vacuum pipe Ø (mm) |
|  |  |  | E=28MeV | E=150MeV |  |  |  |
| BTP000 | BEND (installed) | 260 |  |  | 25 |  | 40 |
| DTP001 | DRIFT | 500 |  |  |  |  |  |
| QTP001 | QUAD | 100 | 2.053 | 0 |  |  |  |
| DTP002 | DRIFT | 500 |  |  |  |  |  |
| QTP002 | QUAD | 100 | -1.730 | 24.2428 |  |  |  |
| DTP003 | DRIFT | 500 |  |  |  |  |  |
| QTP003 | QUAD | 100 | 2.083 | 0 |  |  |  |
| DTP004 | DRIFT | 500 |  |  |  |  |  |
| BTP001 | BEND | 260 |  |  | 25 |  |  |
| DTP005 | DRIFT | 600 |  |  |  |  |  |
| QTP004 | QUAD | 100 | -1.770 | 20.553 |  |  |  |
| DTP006 | DRIFT | 200 |  |  |  |  |  |
| QTP005 | QUAD | 100 | 2.267 | -30.244 |  |  |  |
| DTP007 | DRIFT | 2700 |  |  |  |  |  |
| QTP006 | QUAD | 100 | -1.169 | 7.766 |  |  |  |
| DTP008 | DRIFT | 200 |  |  |  |  |  |
| QTP007 | QUAD | 100 | -1.638 | -27.98 |  |  |  |
| DTP009 | DRIFT | 1200 |  |  |  |  |  |
| TOT length |  | 7860 |  |  |  |  |  |

Thomson final beamline

| Layout Thomson (E= 28 ÷ 150 MeV) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Element | Type | Length (mm) | Field gradient (T/m) |  | Theta (º) | B (T) | Vacuum pipe Ø (mm) |
|  |  |  | E=28MeV | E=150MeV |  |  |  |
| BPT002 | BEND | 260 |  |  | 25 |  |  |
| DTT001 | DRIFT | 330 |  |  |  |  |  |
| QTT001 | QUAD | 100 | -1.15 | 3.461 |  |  |  |
| DTT002 | DRIFT | 330 |  |  |  |  |  |
| QTT002 | QUAD | 100 | 2.75 | 26.082 |  |  |  |
| DTT003 | DRIFT | 330 |  |  |  |  |  |
| QTT003 | QUAD | 100 | -1.15 | 3.461 |  |  |  |
| DTT004 | DRIFT | 330 |  |  |  |  |  |
| BPT003 | BEND | 260 |  |  | 25 |  |  |
| DTT005 | DRIFT | 200 |  |  |  |  |  |
| QTT004 | QUAD | 200 | 1.1464 | 24.888 |  |  |  |
| DTT006 | DRIFT | 100 |  |  |  |  |  |
| QTT005 | QUAD | 200 | -1.5097 | 32.982 |  |  |  |
| DTT007 | DRIFT | 200 |  |  |  |  |  |
| QTT006 | QUAD | 200 | 1.5587 | 2.048 |  |  |  |
| DTT008 | DRIFT | 1650 |  |  |  |  |  |
| STT001 | SOL | 150 |  |  |  | 1.65 | 0 |
| DTT009 | DRIFT | 150 |  |  |  |  |  |
| STT001 | SOL | 150 |  |  |  | 1.40 | 0 |
| DTT010 | DRIFT | 150 |  |  |  |  |  |
| BPT004 | BEND | 260 |  |  | 15 |  |  |
| DTT011 | DRIFT |  |  |  |  |  |  |
| TOT length |  | 6010 |  |  |  |  |  |

Plasmonx final beamline

| Layout Thomson (E= 150 MeV) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Element | Type | Length (mm) | Field gradient (T/m) | Theta (º) | B (T) | Vacuum pipe Ø (mm) |
| BPT003 | BEND | 260 |  | 0 |  | 40 ? |
| DPP001 | DRIFT | 800 |  |  |  |  |
| COLL | COLL | 100 |  |  |  |  |
| DPP002 | DRIFT | 700 |  |  |  |  |
| RFdeflector | RFcav | 260 |  |  |  |  |
| DPP003 | DRIFT | 740 |  |  |  |  |
| BPP001 | BEND | 260 |  | 25 |  |  |
| DPP004 | DRIFT | 330 |  |  |  |  |
| QPP001 | QUAD | 100 | -3.520 |  |  |  |
| DPP005 | DRIFT | 330 |  |  |  |  |
| QPP002 | QUAD | 100 | 46.143 |  |  |  |
| DPP006 | DRIFT | 330 |  |  |  |  |
| QPP003 | QUAD | 100 | -3.520 |  |  |  |
| DPP007 | DRIFT | 330 |  |  |  |  |
| BPP002 | BEND | 260 |  | 25 |  |  |
| DPP008 | DRIFT | 250 |  |  |  |  |
| QPP004 | QUAD | 200 | 0.1179 |  |  |  |
| DPP009 | DRIFT | 100 |  |  |  |  |
| QPP005 | QUAD | 200 | 27.716 |  |  |  |
| DPP010 | DRIFT | 100 |  |  |  |  |
| QPP006 | QUAD | 200 | 29.728 |  |  |  |
| DPP011 | DRIFT | 50 |  |  |  |  |
| QPP007 | QUAD | 200 | 16.748 |  |  |  |
| DPP012 | DRIFT | 250 |  |  |  |  |
| TOT length |  | 6290 |  |  |  |  |

DIPOLES 	5

QUADS 	21

RFdeflector 	1

SOLENOID	2

Steering  Magnets

(coppie)	5 (da calcolare più precisamente)

BPM

Vacuum Pumps

Vacuum pipe

Bellows

Collimator

Element supports

Diagnostics

Controls

Cooling system

Cables

Installation